# Practical No: 01

Aim: Configuring WEP on a Wireless Router

Components: Wireless Router, Router, Switch, Device (PC)

Theory: Wired Equivalent Privacy (WEP) is a security protocol, specified in the IEEE Wireless Fidelity (Wi-Fi) standard, 802.11b. That standard is designed to provide a wireless local area network (WLAN) with a level of security and privacy comparable to what is usually expected of a wired LAN.

Cisco Packet Tracer Setup:- Implementation:

Step 1: Creating connections using Ethernet and serial cable between devices

Step 2: Configuring all devices according to the table below

| Device | Interface | IPv4 address | Other |
| --- | --- | --- | --- |
| PC0 | IP configuration in desktop | 192.168.1.2 | Default Gateway: 192.168.1.1 |
| PC1 | IP configuration in desktop | 192.168.2.2 | Default Gateway: 192.168.2.1 |
| Router0 | FastEthernet0/0 | 192.168.1.1 |  |
| Router0 | FastEthernet0/1 | 20.0.0.1 |  |
| Router0 | Serial0/0/0 | 10.0.0.1 | Clock rate: 64000 |
| Router1 | FastEthernet0/0 | 192.168.2.1 |  |
| Router1 | Serial0/0/0 | 10.0.0.2 | Clock rate: 64000 |
| PC2 | IP Config in desktop |  | Set to DHCP |

Step 3: Configuring wireless router

# Click on wireless router > GUI and set the address

Step 4: Adding security mode as WEP and setting up key as 2a2a2a2a2a

Output:

# Practical No: 02

Aim: Demonstrating Distribution Layer Functions

Components: Router, Switch, Device (PC)

Theory: The distribution layer is the smart layer in the three-layer model. Routing, filtering, and QoS policies are managed at the distribution layer. Distribution layer devices also often manage individual branch-office WAN connections. This layer is also called the Workgroup layer.

Cisco Packet Tracer Setup:-

Implementation:

Step 1: Arranging devices and creating connections using Ethernet and serial cable between devices according to the image above

Step 2: Configuring all devices according to the table below

| Device | Interface | IPv4 address | Other |
| --- | --- | --- | --- |
| PC0 | IP config | 172.16.1.2 | Default Gateway: 172.16.1.1 |
| PC1 | IP config | 192.168.1.2 | Default Gateway: 192.168.1.1 |
| DS_1 | F0/0 | 172.16.1.1 |  |
|  | F0/1 | 10.0.0.1 |  |
|  | S0/0/0 | 30.0.0.1 | Clock rate: 64000 |
|  | RIP v2 | 10.0.0.0 30.0.0.0 172.16.0.0 |  |
| DS_2 | F0/0 | 192.168.1.1 |  |
|  | F0/1 | 20.0.0.1 |  |
|  | S0/0/0 | 30.0.0.2 | Clock rate: 64000 |
|  | RIP v2 | 20.0.0.0 30.0.0.0 192.168.1.0 |  |
| Core_1 | F0/0 | 10.0.0.2 |  |
|  | S0/0/0 | 40.0.0.1 |  |
|  | RIP v2 | 10.0.0.0 40.0.0.0 |  |
| Core_2 | F0/0 | 20.0.0.2 |  |
|  | S0/0/0 | 40.0.0.2 | Clock rate: 64000 |
|  | RIP v2 | 20.0.0.0 40.0.0.0 |  |

Output:

# Practical No: 03

Aim: Placing ACLs

Components: PC, switch, router, server

Theory: A network access control list (ACL) is made up of rules that either allow access to a computer environment or deny it. In a way, an ACL is like a guest list at an exclusive club. Only those on the list are allowed in the doors. This enables administrators to ensure that, unless the proper credentials are presented by the device, it cannot gain access.

Cisco Packet Tracer Setup:-

- Standard ACL

Implementation:

Step 1: Arranging devices and creating connections and assign IP address as shown below

Step 2: creating access-list in router2 CLI as shown below

Output:

- Extended ACL

Implementation

Step 1: Create the layout shown below and set the IP address accordingly

Step 2: configuring the RIP protocol

Step 3: Setting the access list in Router0 > CLI

Output:

# Practical No: 04

Aim: Planning Network-based Firewalls

Components: Wireless Router, Server, PC

Theory: Network firewalls are security devices used to stop or mitigate unauthorized access to private networks connected to the Internet, especially intranets. The only traffic allowed on the network is defined via firewall policies – any other traffic attempting to access the network is blocked.

Cisco Packet Tracer Setup:- Implementation:

Step 1: Arranging devices and creating connections

Step 2: Configure wireless router and connect server to wireless router using Ethernet cable

Step 3: Configure Server by setting IP Config in Server0 to DHCP

Step 4: Configure and connect all PC’s to wireless router

Step 5: Changing port to wireless adapter of all PC’s

After adding wireless adapter of all PC’s they will automatically get connected with wireless router because of DHCP

Step 6: Checking connection of pc’s with server

If receiving response from server our connection is done successfully

Step 7: Configure IPv4 firewall to setup networks based firewall and add conditions

Output:

After the configuration is done for firewall we are unable to ping to server

But we can access the server data (view)

# Practical No: 05

Aim: Configuring Auto Profiles ACU Utilities

Components: WLC (Wireless LAN Controller), AP (Access point), Switch, Server, Laptop, Smartphone, Tablet

Theory: A network access control list (ACL) is made up of rules that either allow access to a computer environment or deny it. In a way, an ACL is like a guest list at an exclusive club. Only those on the list are allowed in the doors. This enables administrators to ensure that, unless the proper credentials are presented by the device, it cannot gain access.

Cisco Packet Tracer Setup: - Implementation:

Step 1: Arranging devices and creating connections

Step 2: WLC (Wireless LAN Controller): Config > Management

IPv4 address: 10.10.10.5	Default Gateway: 10.10.10.1

DNS Server: 10.10.10.2

Step 3: Configuring Laptop and server and checking connection: In Laptop IP Config,

IPv4 address: 10.10.10.10	Default Gateway: 10.10.10.1

DNS Server: 10.10.10.2

In Server0 Config > Fastethernet0,

IPv4 address: 10.10.10.3	Port Status: On In Server0 > Services > DHCP

Interface: (FastEthernet) Service: ON

Default Gateway: 10.10.10.1	DNS Server: 10.10.10.2 Start IP Address: 10.10.10.100	Subnet Mask: 255.0.0.0 Max no. of users: 100

Click on ‘Add’ and then ‘Save’

Check the connection from laptop0 command prompt with C:\> ping 10.10.10.1

Step 4: Configuring Admin settings using address (http://10.10.10.5) in the web browser of Laptop2:

  - Create a new username and password and remember it for further steps

  - In the next page, (Set up your Controller)

System Name: GJCCS	Management IP Address: 10.10.10.5 Subnet Mask: 255.0.0.0	Default Gateway: 10.10.10.1 Management VLAN ID: 0

  - In the Create your Wireless Networks,

Network name: STUDENT	Security: WPA2 Personal

Passphrase: student

  - Click Next in the Advanced Setting section and Apply in the final section

Step 5: Login back to Admin Panel using address (https://10.10.10.5) with the new Admin name and password:

Make sure the 3 Access points are present in the Wireless section,

If not, then re-plug all the access points in the physical section and hit refresh on the top right of the web browser of Laptop0

Go to WLAN make SSID for STUDENT to Student

Step 6: Add new wireless LAN as TEACHER with SSID Teacher and apply and make sure the status is enabled:

Step 7: Create AP Groups for TEACHER and STUDENT:

- In the WLAN tab, select AP Group on the left of the page, below the Advanced section

- Inside, enter

AP Group Name: STUDENT	Description: Student AP And click ‘add’

- In the WLAN section, select WLAN SSID as Student and add

- Go to the APs section and select the first 2 access points and add them to this AP Group

- Now Repeat above the steps for the TEACHER AP Group and in the access point selection, add the one access point that was left out

Step 8: Take Smartphone to connect Student AP group with wireless connection using SSID

Step 9: Take another smartphone to connect Teachers AP group with wireless connection using SSID

Wait for some time (min 30sec to 1min) after that re-plug the adapters of all Access points

Step 10: Send packets from one smartphone to the other

Output:

# Practical No: 06

Aim: Creating an Adhoc Network

Components: Wireless Router, PC

Theory: Ad hoc networks are mostly wireless local area networks (LANs). The devices communicate with each other directly instead of relying on a base station or access points as in wireless LANs for data transfer co-ordination. Each device participates in routing activity, by determining the route using the routing algorithm and forwarding data to other devices via this route.

Cisco Packet Tracer Setup:-

Implementation: -

Step1: Arrange all components i.e., Wireless Router and PC’s

Step 2: Configure wireless routers and connect both of them to each other using Ethernet ports: In Router0, go to GUI > Wireless > basic wireless settings

Network SSID: CS and set SSID broadcast to enabled Now, click on wireless security,

Security Mode: WPA2 Personal, Passphrase: ciscorouter1 Go to the bottom and save settings

In Router1, go to GUI > Wireless > basic wireless settings Network SSID: IT and set SSID broadcast to enabled

Now, click on wireless security,

Security Mode: WPA Personal, Passphrase: ciscorouter2

Step 3: Connect all machines/devices (PC’s) to respective router as per our requirements.

Change the Port of all pc’s with wireless adapter

Configure Wireless connection: Click on PC0 > Desktop > PC Wireless

Click on Connect tab > click on refresh > Select CS/IT > Enter Password and connect

Do similar configuration to all respective PC’s

Output:

# Practical No: 07

Aim: Configuring Basic AP Settings

Components: Router, Access points, PC’s

Theory: A wireless access point (WAP), or more generally just access point (AP), is a networking hardware device that allows other Wi-Fi devices to connect to a wired network. An access point is a device that creates a wireless local area network, or WLAN, usually in an office or large building.

Cisco Packet tracer Setup:

Step 1: Arrange all devices as following

Step 2: Configure Access Points (A)

In Access point CS Port 0, set

Port Status: on, Bandwidth: 100 Mbps, Duplex: Half Duplex Port 1, set

SSID: CS, select WPA2-PSK and password to ciscopacket1

In Access point IT Port 0, set

Port Status: on, Bandwidth: 100 Mbps, Duplex: Half Duplex Port 1, set

SSID: IT, select WPA2-PSK and password to ciscopacket2

Step 3: Configure and Setup IP Address for all devices (PC’s)

| Device | Interface | IPv4 address | Other |
| --- | --- | --- | --- |
| CS-1 | IP config | 192.168.1.3 | Default Gateway: 192.168.1.1 |
| CS-2 | IP config | 192.168.1.4 | Default Gateway: 192.168.1.1 |
| IT-1 | IP config | 171.16.10.2 | Default Gateway: 171.16.10.1 |
| IT-2 | IP config | 171.16.10.3 | Default Gateway: 171.16.10.1 |
| CS-1 and CS-2 | Wireless0 |  | SSID: CS WPA2-PSK password: ciscopacket1 |
| IT-1 and IT-2 | Wireless0 |  | SSID: IT WPA2-PSK password: ciscopacket2 |
| Router2 | F0/0 | 192.168.1.1 |  |
|  | S0/0/0 | 20.0.0.1 |  |
|  | RIP v2 | 20.0.0.0 192.168.1.0 |  |
| Router3 | F0/0 | 171.16.10.1 |  |
|  | S0/0/0 | 20.0.0.2 |  |
|  | RIP v2 | 20.0.0.0 171.16.10.0 |  |

Note: Change all port adapters with wireless adapter for all PC’s

Output:

# Practical No: 08

Aim: Configure fast Ethernet on router using packet tracer

Components: Router, Switches, PC’s

Theory: Fast Ethernet is used for departmental backbones, connections to high-speed servers, and connections to workstations running bandwidth-intensive software such as CAD or multimedia applications.

Cisco Packet tracer Setup:

Implementation:

Step 1: Arrange all devices as shown below:

Step 2: Configure Router using CLI, using following commands: configure t

hostname R1

enable password cisco interface fa0/0

ip address 192.168.2.1 255.255.255.0

no shutdown exit

Exit

Step 3: Configure All PC’s and check the connection.

| Device | Interface | IPv4 address | Other |
| --- | --- | --- | --- |
| PC0 | IP config | 192.168.2.2 | Default Gateway: 192.168.2.1 |
| PC1 | IP config | 192.168.2.3 | Default Gateway: 192.168.2.1 |
| PC2 | IP config | 192.168.2.4 | Default Gateway: 192.168.2.1 |

Step 4: Ping 192.168.2.1 with all the PCs.

Output:

# Practical No: 09

Aim: Configure Site-to-Site Wireless Link

Components: Routers, Switch, PC

Theory: A site-to-site VPN connects users in different locations within an entire network. Through this network, the users can exchange data from their own locations while that information is encrypted and secured through the VPN. Users working in separate offices can still be connected to one another and all of their internal resources. This keeps all users connected even when they are working remotely while securing the information exchanged between them.

Cisco Packet tracer Setup:

Step 1: Connect all devices and assign IP addresses as shown below

Step 2: Set IP route as shown below in the ISP router

Step 3: Try pinging from PC0 or PC1 to check the connection.

In Desktop, Command prompt C:\> ping 192.168.10.10

Step 4: Configure iskamp policy and define IPsec transform set by entering the commands given below in the HQ router

Step 5: Create access list and crypto map in the HQ router CLI

Step 6: Perform the above 2 steps for the Branch router and change the IP addresses accordingly

Output:

Verify the created isakmp policies with the following command